Code- :

smart helmet accident for detection

Smart helmet for accident detection using wifi , recived msg when accident is detected*/

#include <Wire.h>

#include <ESP8266WiFi.h>

#include <WiFiClientSecure.h>

const char* ssid = "Pgl.❤️🥰";

const char* password = "MH050553";

String botToken = "8354725551:AAHKeKvUvCn95ZoeZE3BhAIwHD2vNZeNUDs";

String chatID = "8570744271";

WiFiClientSecure client;

const int buzzer = D5;

const int button = D7;

const int MPU_addr = 0x68;

int16_t AcX, AcY, AcZ;

bool accidentSent = false;

bool alarmDisabled = false;

int triggerCount = 0;

// double tap variables

unsigned long lastTapTime = 0;

int tapCount = 0;

float tapThreshold = 2.2;   // adjust sensitivity

// universal buzzer function (works for all buzzers)

void beep(int duration){

unsigned long start = millis();

while(millis() - start < duration){

digitalWrite(buzzer,HIGH);

delayMicroseconds(500);

digitalWrite(buzzer,LOW);

delayMicroseconds(500);

}

}

// telegram function

void sendTelegram(String message) {

client.setInsecure();

if (!client.connect("api.telegram.org", 443)) {

Serial.println("Connection failed");

return;

}

String url = "/bot" + botToken + "/sendMessage?chat_id=" + chatID + "&text=" + message;

client.print(String("GET ") + url + " HTTP/1.1\r\n" +

"Host: api.telegram.org\r\n" +

"Connection: close\r\n\r\n");

}

void setup() {

Wire.begin(D2, D1);

Serial.begin(9600);

pinMode(buzzer, OUTPUT);

pinMode(button, INPUT_PULLUP);

// MPU6050 start

Wire.beginTransmission(MPU_addr);

Wire.write(0x6B);

Wire.write(0);

Wire.endTransmission(true);

// WiFi connect

WiFi.begin(ssid, password);

Serial.print("Connecting WiFi");

while (WiFi.status() != WL_CONNECTED) {

delay(500);

Serial.print(".");

}

Serial.println("Connected!");

}

void loop() {

// read sensor

Wire.beginTransmission(MPU_addr);

Wire.write(0x3B);

Wire.endTransmission(false);

Wire.requestFrom(MPU_addr, 6, true);

AcX = Wire.read() << 8 | Wire.read();

AcY = Wire.read() << 8 | Wire.read();

AcZ = Wire.read() << 8 | Wire.read();

float x = AcX / 16384.0;

float y = AcY / 16384.0;

float z = AcZ / 16384.0;

Serial.print("X: "); Serial.print(x);

Serial.print(" Y: "); Serial.print(y);

Serial.print(" Z: "); Serial.println(z);

float magnitude = sqrt(x*x + y*y + z*z);

// ---------- DOUBLE TAP DETECT ----------

if (magnitude > tapThreshold) {

unsigned long currentTime = millis();

if (currentTime - lastTapTime < 500) {

tapCount++;

} else {

tapCount = 1;

}

lastTapTime = currentTime;

if (tapCount == 2) {

alarmDisabled = !alarmDisabled;

Serial.println("DOUBLE TAP DETECTED");

beep(400);

tapCount = 0;

delay(300);

}

}

// ---------- BUTTON DISABLE ----------

if (digitalRead(button) == LOW) {

alarmDisabled = true;

Serial.println("Alarm Disabled by Button");

delay(500);

}

// ---------- ACCIDENT DETECT ----------

if (!alarmDisabled) {

if (abs(x) > 0.50 || abs(y) > 0.50) {

triggerCount++;

} else {

triggerCount = 0;

}

// confirm accident

if (triggerCount >= 3) {

beep(1000);

if (!accidentSent) {

sendTelegram("🚨 Accident Detected!");

accidentSent = true;

}

} else {

accidentSent = false;

}

}

delay(150);

}